# ACI205 Mifare Reader

ADVIC32.DLL Dynamic Link Library

Operation Manual

The application will operate the ACI Mifare Reader with the DLL ADVIC32.DLL. The DLL’s duty is to transform the user’s operating of the card into detailed communication protocol, then translate it into the command for operating the card. ADVIC32.DLL is called through dynamic.

The follow is detail description for the ADVIC32.DLL functions with Mifare card.

- Operation System

32 bits WINDOWS 95/98/2000/XP

- Function List

ADVIC32.DLL include the follow functions for the Mifare card operation.

2.1) General Function

long pascal AutoLinkCom(long Port);

long pascal LinkCom(long Port);

long pascal UnLinkCom(void);

long pascal ALARM(long mode,long actiontime,

long intervaltime,long count);

long  pascal RFMifare_Status(void);

long  pascal RFMifare_Mcmno(long * mcmno);

long  pascal RFMifare_Mcmver(unsigned char * mcmver);

2.2) Basic card operate function

long  pascal RFMifare_LoadKey(long mode,long sector,unsigned char *mima);

long  pascal RFMifare_Request(long mode,long *cardtype);

long  pascal RFMifare_AntiColl(long *snr);

long  pascal RFMifare_Select(long snr);

long  pascal RFMifare_Authentication(long mode ,long key_adr ,long

blockno);

long  pascal RFMifare_Read(long blockno , unsigned char * readbuff);

long  pascal RFMifare_Write(long blockno , unsigned char *writebuff);

long  pascal RFMifare_Increment(long blockno , long value);

long  pascal RFMifare_Decrment(long blockno , long value);

long  pascal RFMifare_Halt(void);

2.3) Advance card operate function

long pascal RFMifare_TransKey(long sector , unsigned char *mima);

long pascal RFMifare_Authmode(long authmode);

long pascal RFMifare_GetType(long * cardtype);

long pascal RFMifare_Getsnr(long *snr);

long pascal RFMifare_ReadBlock(long blockno , long snr , unsigned char

*dat);

long pascal RFMifare_WriteBlock(long blockno,long snr,unsigned char

*databuff);

long pascal RFMifare_WriteKey(long sector,long snr,unsigned char *mima )

long pascal RFMifare_Readvalue(long blockno,long snr,long *value);

long pascal RFMifare_Writevalue(long blockno , long snr , long *value);

long pascal RFMifare_Incrementvalue(long blockno,long snr,long *value);

long pascal RFMifare_Decrementvalue(long blockno,long snr,long *value);

- Description of The Function

3.1) General function

3.1.1) AutoLinkCom() : Auto connect with COM

Function description:

This function is used to recognize the serial port automatically, which is connected with the reader, and carry on the initial operation, then create communication linkage by connecting the serial port with the reader. Transport protocol is Baud Rate 9600 BPS, 8 bits,1 stop bit, no odd/even bit.

You must connect the COM first before call the other function.

Usage:

long AutoLinkCom(long * port)

Parameter:

Port: COM1—COM10: which is used to return serial port number connected with the reader.

The definitions of the constants COM1-COM10 have been given in the document of advic32.h.

Returns:

The return value is zero, if the function is successful.

Otherwise, it returns negative value containing an error code.

Examples: retcode=AutoLinkCom(&port);

Reference: LinkCom()

3.1.2) LinkCom()：Connect with COM

Function description:

This function is used to appoint the serial port initially, and create the communication linkage by connecting the serial port with   the reader. Transport protocol is Baud Rate 9600 BPS, 8 bits,1 stop bit, no odd/even bit.

You must connect the COM first before call the other function.

Usage:

long AutoLinkCom(long port)

Parameter:

Port: COM1—COM10: The definitions of the constants COM1-COM10 have been given in the document of advic32.h.

Returns: The return value is zero, if the function is successful.

Otherwise, it returns negative value containing an error code.

Examples: retcode=LinkCom(COM1);

Reference: AutoLinkCom()

3.1.3) UnLinkCom(): Disconnect the serial port

Function description:

This function is used to release the serial port that has been connected and dismiss the communication linkage between the serial port and the reader. In some development environment, the source of serial port must be released before exiting from the program, otherwise it my cause a breakdown of the system.

Usage:

long UnLinkCom()

Parameter:none

Returns: The return value is zero, if the function is successful.

Otherwise, it returns negative value containing an error code.

Examples: retcode=UnLinkCom();

Reference: AutoLinkCom(),LinkCom()

3.1.4) ALARM()：Control the LED & BEEP active

Function description:

This function is used to control the Reader’s LED & Beep active.

You can control these device in open state,active count,and active time.

Usage:

long pascal ALARM(long mode,long actiontime,long intervaltime,long

count)

Parameter:

Mode: device number, the value is the device (REDLED,GREENLED,

BELL) OR value ,which you would control .

Example: if you control the REDLED and GREENLED active

mode=REDLED|GREENLED

actiontime: the time for device is on state. The unit is 10ms.

Example: actiontime=15, the actual time is 150ms.

Intervaltime: the time for device is off state,The unit is 10ms.

count: The counts of the device is on/off .

Returns: The return value is zero, if the function is successful.

Otherwise, it returns negative value containing an error code.

Examples: retcode=ALARM(mode,actiontime,intervaltime,count);

Reference:

3.1.5) RFMifare_Status() : Inquire the Status of the Reader

Function description:

This funcion is used to inquire the reader status is in connecting status.

Usage:

long pascal RFMifare_Status(void)

Parameter:none

Returns: The return value is zero, if the function is successful.

Example: retcode=RFMifare_Status();

3.1.6 ) RFMifare_Mcmno(long *mcmno): Read the reader device ID

Function description:

This function is used to read the reader device ID.

Usage: long pascal RFMifare_Mcmno(&mcmno)

Parameter:

Mcmno: the reader returns its device ID.

Returns: The return value is zero, if the function is successful.

Example: long mcmno;

retcode=RFMifare_Mcmno(&mcmno);

3.1.7 ) RFMifare_Mcmver(unsigned char *mcmver): Read the reader’s control software version.

Function description:

This function is used to read the reader’s control software version.

Example: the software version is VER1.00A.

Usage:

long pascal RFMifare_Mcmver(Unsigned char *mcmver)

Parameter:

mcmver: the control software version (8 bytes).

Returns: The return value is zero, if the function is successful.

Example:char mcmver[8];

retcode=RFMifare_Mcmver(mcmver);

3.2) Basic card operate function

3.2.1) RFMifare_LoadKey(long mode,long sector,unsigned char *mima);

download the card sector key

Function description:

This function is used to download a card sector key to reader , the key would save in EEPROM of the reader..

Usage: long pascal RFMifare_LoadKey(long mode,long sector,unsigned

char *mima);

Parameter:

mode  :  download KEYA or KEYB

mode=KEYA ; Download the KEYA

mode=KEYB ; Download the KEYB

sector: the sector number for download the key

mima: download the key (6 bytes)

Returns: The return value is zero, if the function is successful.

Example:char mima[8];

retcode= RFMifare_LoadKey(KEYA,1,&mima);

3.2.2) RFMifare_Request(long mode,long *cardtype); Request a card

Function description:

This function is used to request a card. If the card is ack, it will return the card type.

Usage:

long  pascal RFMifare_Request(long mode,long *cardtype);

Parameter:

mode: request mode,

mode=ALL;  request all card

mode=IDLE; request all card ignore the card in HALT state.

cardtype：card type

cardtype = 0x0010   Mifare Light 10

cardtype = 0x0004   Mifare one

Returns: The return value is zero, if the function is successful.

3.2.3) RFMifare_Anticoll(long *snr); Card anticoll

Function description:

This function is used to anticoll the card, and return the card serial number.

In this reader, you can only operate one card .

Usage:

long  pascal RFMifare_AntiColl(long *snr);

Parameter:

Snr: the card return its serial number.

Returns: The return value is zero, if the function is successful.

3.2.4) RFMifare_Select(long snr); Select the card

Function description:

This function is used to active the card, you must select the card before you operate the card.

Usage:

long  pascal RFMifare_Select(long snr);

Parameter:

snr: sector card’s serial number, the snr is return Anticoll function.

Returns: The return value is zero, if the function is successful.

3.2.5) RFMifare_Authentication(long mode ,long key_adr ,long blockno);

Card authentication

Function description:

This function is used to authentication a card KEYA or KEYB.

You must authenticate the card before read/write the card.

Usage:

long pascal RFMifare_Authentication(long mode,long key_adr,long

blockno);

Parameter:

mode  : authenticate with KEYA or KEYB

mode=KEYA;    authenticate with KEYA

mode=KEYB;    authenticate with KEYB

key_adr  :  KEY’s sector number

blockno  ： KEY’s block number.

Example: if you authenticate the sector 1,the blockno is 7.

Returns: The return value is zero, if the function is successful.

3.2.6 RFMifare_Read(long blockno,unsigned char *readbuff)

## Read the card data

Function description:

This function is used to read the card data (16 bytes).

Usage:

long pascal RFMifare_Read(long blockno , unsigned char * readbuff);

Parameter:

blockno  : block number (0—63)

readbuff : data buffer

Returns: The return value is zero, if the function is successful.

3.2.7) RFMifare_Write(long blockno , unsigned char *writebuff)

Write the card data

Function description:

This function is used to write the data to the card(16 bytes).

Usage:

long  pascal RFMifare_Write(long blockno , unsigned char *writebuff);

Parameter:

blockno  : block number (0-63)

writebuff: write data buffer

Returns: The return value is zero, if the function is successful.

3.2.8) RFMifare_Increment(long blockno , long value)

Increment the value for card “Electronic Purse”.

Function description:

This function is used to increment the value for card EP.

Usage:

long  pascal RFMifare_Increment(long blockno , long value);

Parameter:

blockno : EP’s block number

value  : increment the value for EP

Returns: The return value is zero, if the function is successful.

3.2.9) RFMifare_Decrment(long blockno , long value);

Descrement the value from card “Electronic Purse”

Function description:

This function is used to descrement value from the card EP.

Usage:

long  pascal RFMifare_Decrment(long blockno , long value);

Parameter:

blockno: EP’s block number

value   : decrement the value from the card EP

Returns: The return value is zero, if the function is successful.

3.2.10) RFMifare_Halt(void) : Halt the card

Function description:

This function is used to halt the card, and card is in idle state.

Usage:

long  pascal RFMifare_Halt(void);

Parameter: none

Returns: The return value is zero, if the function is successful.

3.3) Advance card operate function

The advance function is combined with  many basic card operate functions. For example, the read card function is combined with request,anticoll,select,authentication and read operate.The operate could simple the user operate.

In advance function, we only use KEYA.

3.3.1) RFMifare_TransKey(long sector , unsigned char *mima)

Download KEY

Function description:

This function is used to download the KEYA to reader.

In advance function,we only use KEYA.

Usage:

long  pascal RFMifare_TransKey(long sector , unsigned char *mima);

Parameter:

sector : the download KEY sector number(0-15)

mima : download the KEYA

Returns: The return value is zero, if the function is successful.

3.3.2) RFMifare_Authmode(long authmode): Set the KEY authentication mode.

Function description:

This function is used to set the KEY authentication mode,this function is only used to ML10 card.

Usage:

long  pascal RFMifare_Authmode(long authmode);

Parameter:

Authmode: KEY authentication mode

Authmode=0: use the KEYA

=1: use the KEYB

Returns: The return value is zero, if the function is successful.

3.3.3) RFMifare_GetType(long * cardtype) :Read card type

Function description:

This function is used to read the card type.

Usage:

long  pascal RFMifare_GetType(long * cardtype);

Parameter:

cardtype: Card type

cardtype=0x0004   Mifare 1 card

cardtype=0x0010   Mifare Light 10 card

Returns: The return value is zero, if the function is successful.

3.3.4) RFMifare_Getsnr(long * snr):Read card serial number

Function description:

This function is used to read the card serial number.

Usage:

long  pascal RFMifare_Getsnr(long *snr);

Parameter:

snr: card serial number

Returns: The return value is zero, if the function is successful.

3.3.5) RFMifare_ReadBlock(long blockno ,long snr , unsigned char

*dat);

Read card data

Function description:

This function is used to read data from a card.

Usage:

long  pascal RFMifare_ReadBlock(long blockno , long snr , unsigned

char *dat);

Parameter:

blockno : The block number which you would read

snr  : card serial

dat  : data buffer (16 bytes)

Returns: The return value is zero, if the function is successful.

3.3.6) RFMifare_WriteBlock(long blockno,long snr,unsigned char

*databuff);

Write data to card

Function description:

This function is used to write data (16 bytes) to the card.

If successful, return the data from the card.

Usage:

long pascal RFMifare_WriteBlock(long blockno,long snr,unsigned char

*databuff);

Parameter:

blockno    : data block number

snr    : card serial number

databuff    : data buffer (16 bytes)

Returns: The return value is zero, if the function is successful.

3.3.7) RFMifare_WriteKey(long sector,long snr,unsigned char *mima)

Modify card KEYA

Function description:

This function is used to modify the card KEYA.

Usage:

long pascal RFMifare_WriteKey(long sector,long snr,unsigned char

*mima, unsigned char ctr)

Parameter:

Sector : sector number

snr : card serial number

mima : new KEYA ( 6 bytes)

ctr : control bit (0xFF)

Returns: The return value is zero, if the function is successful.

3.3.8) RFMifare_Readvalue(long blockno,long snr,long *value);

Read the card “Electronic Purse”

Function description:

This function is used to read the card EP.

Usage:

long pascal RFMifare_Readvalue(long blockno,long snr,long *value);

Parameter:

snr  : card serial number

blockno  : EP block number

value  : the value of EP

Returns: The return value is zero, if the function is successful.

3.3.8) RFMifare_Writevalue(long blockno , long snr , long *value)

Write “Electronic Purse”

Function description:

This function is used to write a value to the card EP.

Usage:

long  pascal RFMifare_WriteValue(long blockno , long snr ,

long *value);

Parameter:

snr  : card serial number

blockno  : EP block number

value  : write value

Returns: The return value is zero, if the function is successful.

3.3.9) RFMifare_Incrementvalue(long blockno,long snr,long *value)

Increment value to “Electronic Purse”

Function description:

This function is used to increment value to EP.

Usage:

long pascal RFMifare_Incrementvalue(long blockno,long snr,long

*value);

Parameter:

snr  : card serial number

blockno: EP block number

value  : value which to increment EP

Returns: The return value is zero, if the function is successful.

3.3.10)RFMifare_Decrementvalue(long blockno,long snr,long *value)

Decrement value from “Electronic Purse”

Function description:

This function is used to decrement a value from the card EP.

Usage:

long pascal RFMifare_Decrementvalue(long blockno,long snr,long *value);

Parameter:

snr  : card serial number

blockno  : EP block number

value  : the value which decrement from EP.

Returns: The return value is zero, if the function is successful.

- Definition

#define REDLED       1      /* RED LED DRIVER NUMBER */

#define GREENLED     2      /* GREEN LED DRIVER NUMBER*/

#define BELL         4      /* BELL DRIVER NUMBER */

#define IDLE         0

#define ALL          1

#define MI_OK        0

#define MI_ERR       1

#define KEYA         0        /* KEY A */

#define KEYB         4        /* KEY B */