# ACM.DLL Specification

The information contained in this email / document is confidential. You can only use the information for internal company use.  You or your colleague cannot use, copy, disclose or deliver this to anyone else.  ** End of Disclaimer **

1. long AddSortedCardA(int InsertPos, unsigned long CardID, long* GroupA, TCardType CardType, long Password, char* StartDate,  char* EndDate,  bool IsEnd);

Description: Add a Sorted Card to controller, Add sorted card directly write a card in the memory directly to controller. Suffix “A” means Part one of card.

Parameter: 	InsertPos(0-20000)

CardID (0x00000000-0xFFFFFFFF)

GroupA Point to an long array that contain Group number(0-63), e.g.:

long Group[]={0,1,2,3,4,5,6,7,8,9,10,11,12,13,14,15,16,17,18,19}; //OK

long Group[]={1,2,3,255, 255, 255, 255, 255, 255, 255, 255, 255, 255, 255, 255, 255, 255, 255, 255, 255} //OK

Note: Total number of element is 20. If no group, fill 0xFF; the group number must in Ascending order.

Enum TCardType{ctRegularCard=0, ctTempCard=1}

Password(0-9999)

StartDate, EndDate Point to a null-terminate string in YYMMDD, e.g.:

Char* startdate=”041001”; // 1st OCT 2004

If the Card is the last, IsEnd must set to true(non-zero)

Return Value:	1 — Success;

0 –- Fail;

2. int AddSortedCardB(unsigned int InsertPos, long* GroupB):

Parameter: 	InsertPos(0-20000)

GroupB same as GroupA in AddSortedCardA

long Group[]={0,1,2,3,4,5,6,7,8,9,10,11,12,13,14,15,16,17,18,19}; //OK

Description: You need this only if you are using Controller Version 4 (20 Groups Version). Must be used for the same card with AddSortedCardA when using Multi-Group controller.

Return Value:	1 — Success;

0 –- Fail;

3. int AddUnSortedCardA(unsigned long CardID, long* GroupA, TCardType CardType, long Password, Char* StartDate, Char* EndDate);

Description: Write a card to controller, and controller add this card after the END of SortedCard. Controller will search the card automatically, if the card already existed in SortedCardList, controller will update that card, and therefore not in UnSortedCardList.

Parameter: See AddSortedCardA

Return Value:	1 — Success;

0 –- Fail;

4. int AddSortedCardB(unsigned long CardID, long* GroupB):

Parameter: 	CardID(0x00000000-0xFFFFFFFF)

GroupB same as GroupA in AddSortedCardA

Description: You need this only if you are using Controller Version 4 (20 Groups Version). Must be used for the same card with AddUnSortedCardA when using Multi-Group controller.

Return Value:	1 — Success;

0 –- Fail;

5. Bool Connect(void);

Description: Connect to Controller, please set “COM port” and “ControllerID” before use this function

Return: True – connect successful

False –Connect fail

6. Void Disconnect(void);

Description: Disconnect controller

Return: N/A

7. int ReadTransaction (unsigned char *buffer);

Description : collect transaction log and store in buffer [0 – 3]

Remark : buffer[0~3] — Card number（Higher byte at front）;buffer[4~9] –- Year, Month, Day, Hour, Minute, Second; buffer[10] — Event（0 — None; 1 — Legal entrance; 2 — double entrance; 3 — double entrance and without door open, Anti Pass Back; 4 — No Time; 5 — Legal entrance with open door; 6 — double exit;7 — double exit without door open, Anti Pass Back; 8 — No permit (Door); 9 — No permit (Card); 10 — Illegal door open; 14- Invalid PIN, 16- Release Button, 17 – Break in, 18- Door Open Timeout, 19- Void Card, 20- Expired Card, 21- Going to void）buffer[11] — Door Number（1~16）; buffer[12~13] — Sequence Number

Parameter : buffer;

Return Value :	1 : 	Success;

0 : 	Fail;

-1 : 	No response;

8. int ResetController(void);

Description : System reset; After reset, all setting and memory will be cleared. Date and Time will be also reset to 1/1/2000 00:00.

Parameter : N/A;

Return Value :	1 : 	Success;

0 : 	Fail;

-1 : 	No response;

9. int SendGroup(int group, unsigned long door_ctrl, char *tmtablnum);

Description: Set Access Group - group（0~63）, door_ctrl is a control code for the 16 doors（bit0~15 represent door 1~16; doors enable with relative bit = 1 and disable with relative bit = 0 ）, tmtablnum[0]-[3] are respect to the relative door’s Time Zone ( 0~31 ), Anti Pass Back, Password and Holiday control ( tmtablnum[0] for Door 1 ……tmtablnum[15] for Door 16). Bit7 is use for Anti Pass Back (0 ~ Disable, 1 ~ Enable), Bit 6 is use for Password function (0 ~ Disable, 1 ~ Enable), Bit 5 is use for Holiday control (0 ~ Disable, 1 ~ Enable). Bits 0~4 are Time Zone number for that Door.

Parameter : Access Group Number（0~63）, Control code of 16 Door, Time Zone Table ( 0~31 ), Anti Pass Back, Password and Holiday control.

Return Value :	1 : 	Success;

0 : 	Fail;

-1 : 	No response;

10. int SendHoliday(byte holidayid;  month: Byte; day: Byte;

HolidayType:Byte);

Description : set holiday table

Remark : Type of Holiday - type（1: Holiday Type 1,）;  // later we will add Type 2

Parameter : holiday id (0-59), mm, dd, holiday type;

Return Value :	1 : 	Success;

0 : 	Fail;

-1 : 	No response;

11. int SendPCTime (void);

Description : send PC time to controller;

Parameter : N/A

Return Value :	1 : 	Success;

0 : 	Fail;

-1 : 	No response;

12. int SendPinDisableTimeTable (char *tmtablnum);

Description : disable PIN entry

Remark : tmtablnum[0]-[15] is for door 1 – 16,

Parameter : Time Zone number;

Return Value :	1 : 	Success;

0 : 	Fail;

-1 : 	No response;

13. int SendSetControllerID(long NewID, Char* SN);

Description : Set Master Controller to new id according to Controller’s SN

Parameter : New controller id(1-16); Controller’s SN(eg “12345678”)

Return Value :	1 : 	Success;

0 : 	Fail;

-1 : 	No response;

14. int SendSetDeviceID(byte NewDeviceID, TDeviceType DeviceType,Char* DeviceSN);

Description : Set Reader or Door Controller ID via its connected Master Controller

Parameter: NewDeviceID(1-32)

Enum TDeviceType{ dtReader=0x41, dtDoorSlave=0x44);

DeviceSN: 8char string(eg. “12345678”) with null-terminate

Return Value :	1 : 	Success;

0 : 	Fail;

-1 : 	No response;

15. Int SendTimeTable(byte tmtablnum, char* tmzonebuff);

Description : write time table tmtablnum from buffer tmzonebuff;

Parameter : time table number (0-31) , time zone buffer

Remark : tmzonebuff (0 - 6) Mon – Sun=>Time zone (0 - 31)

tmzonebuff[7] => Time Zones ( 0~31) of Holiday Type 1

tmzonebuff[8~10] => Reversed = 0;

Return Value :		1 : 	Success;

0 : 	Fail;

-1 : 	No response;

16. int SetTimeZone(byte tmzonenum, long totalzone, long *zonebuff);

Description : set time zone tmzonenum (0~31) , Total time segments for this Time Zone is totalzone（0~287）, zonebuff use to store the time segment’s details. Time segment contains 2 elements – one is starting time and other is ending time. These 2 elements are representing in minutes and minimum value is 5 minutes. For example: Time Zone number 4 with these 3 time segments - 8:00~11:30, 14:00~17:00, 19:00~22:00, So, zonebuff[0]=480, zonebuff[1]=690, zonebuff[2]=840, zonebuff[3]=1020, zonebuff[4]=1140, zonebuff[5]=1320, totalzone =3, tmzonenum=4;

Parameter : controller id,  Time Zone number ( 0~31), Total number of Time Segments（0~287）, Time Segment’s details;

Parameter : time zone number (0-31) , time (0-287), time info

Return Value :	1 : 	Success;

0 : 	Fail;

-1 : 	No response;

17. int SetDoorOpenTime(Byte door, Byte OpenSec, Byte AlarmSec);

Description : Set door open time and sensor monitor time out time;

Parameter : Door, OpenSec: relay activate sec(0-255), AlarmSec: Alarm monitor time out sec (0-255)

Return Value :	1 : 	Success;

0 : 	Fail;

-1 : 	No response;

//------------------------------------- DLL Setting functions ------------------------------

18. Void SetConnectMethod( TConnectMethod method);

Parameter: enum TConnectMethod{cmSerial=0, cmTCPIP=1}

19. Void SetControllerID(Byte ID);

Parameter: set current communicate controller1-16 controller ID(0 means broadcast)

20. Void SetCOM(int Value); //Serial only

Parameter: Value is COM port, e.g. 1=COM1, 2=COM2

21. Void SetReadTimeOut(long timeout); //Serial only

Parameter: Set Serial Time-out in millisecond. Should be 100~300

22. Void SetHost(char* Value); //TCP/IP only

Parameter: URL string for the TCP/IP Host (eg. “192.168.1.1”, “micosoft.com”)

23. Void SetHostPort(unsigned int Value); //TCP/IP only

Parameter: Host Port (1-65535)

24. Void SetRetrySleep(long retrysleep);//TCP/IP only

Parameter: retrysleep- delay retrysleep in millisecond between each retry (should be 0~100)

25. Void SetRetryTime(long retrytime);//TCP/IP only

Parameter: retrytime – retry retrytime times(should be 10~20) to receive a complete command

Note: ReadTransaction Function can’t be call too fast, for the best result, delay 0.2sec between ReadTransaction.

Others Communication functions have NO delay required.